- Copy the .tar file to a USB drive from computer. (use empty drive  and format it to FAT32 before transfer)

- Plug in the drive to the EVM USB port

- Plug in the USB drive

Lsblk -> to check if USB is connected.

You should see something like /dev/sda1 or /dev/sdb1 — that’s your USB.

- Mount the USB drive

mkdir -p /mnt/usb
mount /dev/sda1 /mnt/usb

ls /mnt/usb

- Copy your model files to the board

- Untar the file into a folder.

- Say your model is in /mnt/usb/model_folder, copy it to your device’s model folder:

cp -r /mnt/usb/dr.tar /opt/model_zoo/

- Confirm it’s there:

ls /opt/model_zoo/

To untar in model_zoo:

tar -xvf dr.tar

- Update your YAML or config

- Point the model path in your run.yaml to the new location:

models:
  model0:
    model_path: "/opt/model_zoo/cl-6090"

Example .yaml file:

Filename: run.yaml (under model folder)

title: "DR Classification Demo"

flows:

flow0:

- cam_in

- model0

- display

inputs:

cam_in:

source: "/dev/video-usb-cam0"

width: 1280

height: 720

framerate: 30

models:

model0:

model_path: "/opt/model_zoo/dr"

task_type: classification

top_k: 5

outputs:

display:

sink: kmssink

width: 1280

height: 720

- Run the model

- Navigate to the apps_python folder

cd /opt/edgeai-gst-apps/apps_python

python3 app_edgeai.py /opt/model_zoo/dr/run.yaml

(change foldername according to your model and device)

v4l2-ctl --list-devices

End-to-End Workflow

import subprocess import threading

def wait_for_input(process):

input(" Press ENTER to switch to next model...\n")

process.terminate()

def run_model(config_path, log_file):

print(f"\n Running: {config_path}\n")

process = subprocess.Popen(
    ["python3", "app_edgeai.py", config_path],
    cwd="/opt/edgeai-gst-apps/apps_python",
    stdout=subprocess.PIPE,
    stderr=subprocess.STDOUT,
    text=True
)

def log_output():
    with open(log_file, "w") as f:
        for line in process.stdout:
            print(line, end="")   # show in terminal
            f.write(line)        # save to file

t1 = threading.Thread(target=log_output)
t2 = threading.Thread(target=wait_for_input, args=(process,))

t1.start()
t2.start()

process.wait()
t1.join()
t2.join()

print(f"\nStopped: {config_path}")
print(f"Output saved to: {log_file}\n")

if name == "main":

run_model("/opt/model_zoo/dr/run.yaml", "dr_output.log") run_model("/opt/model_zoo/cataract/run.yaml", "cataract_output.log")